| OPERATIONAL MANUAL |  |  |  |
| --- | --- | --- | --- |
| #### Windows 95 / 98 / NT ### Communication ## Driver # TWSWTCP.EXE |  |  |  |
|  | - EDITION |  |  |
|  |  | Month | Year |
|  | 1st | September | 1998 |
|  | 2nd | July | 1999 |
|  | 3rd | August | 2000 |
|  | 4th | May | 2001 |
| TERAOKA WEIGH-SYSTEM PTE LTD 4 LENG KEE ROAD , #05-03/04/05 , SIS BUILDING , SINGAPORE 159088. TEL: (65) 472 2996 / 378 5361 FAX: (65) 472 2869 |  |  |  |

CONTENTS

- SM-90 WORKSTATION CONFIGURATION..……………………………3

- WINDOWS 95 / NT & SM-90 COMMUNICATION

2.1 ETHERNET ADAPTER CARD………………………………………..4

2.2 HOSTS…………………………………………………………………..4

2.3 SCALE PORT NUMBER..……………………………………………..4

2.4 SM-90 FILES SUPPORTED BY TWSWTCP.EXE……………………5

2.5 ERROR CODES GENERATED BY TWSWTCP.EXE….…………….6

2.6 UPLOAD PLU ITEM FILE……………………………………………..7

2.7 UPLOAD PLU DELETE FILE………………………………………….9

2.8 DOWNLOAD PLU ITEM FILE……………………………………….10

2.9 DOWNLOAD PLU TOTAL FILE……………………………………..11

2.10 UPLOAD PRESET KEY ASSIGNMENT FILE…..…………………13

- SM-90 FILE STRUCTURE

3.1   CODE TYPE……..………….………………………………………...14

3.2   DEPARTMENT.………………...…………………………………….15

3.3   DEPARTMENT TOTAL……………...………………………………15

3.4   MAIN GROUP…………..………………...…………………………..16

3.5   MAIN GROUP TOTAL…….…………………...…………………….16

3.6   PLU…………………………………………………...………………..17

3.7   PLU TOTAL…………………………………………………………...25

3.8   PRINT FORMAT…………………………………………..….………26

3.9   TEXT……………………………………………………………..……35

3.10 INGREDIENT…………………………………………………….…...35

3.11 SPECIAL MESSAGE………………………………….………………35

3.12 SHOP NAME………………………………………………………….36

3.13 SCROLL MESSAGE…………………….…………………………….36

3.14 SCROLL SEQUENCE………….……………………………………...37

3.15 PRESET KEY ASSIGNMENT………………………………………..38

3.16 STORE TOTAL………………………………..………………………40

SM-90 WORKSTATION CONFIGURATION WITH PC (ETHERNET INTERFACE)

Workstation #1    Workstation #2     Workstation #3

………..

PC

HUB

NOTE :	Maximum of 255 SM-80SX / SM-90 scales can be connected via the above

configuration and an Ethernet Adapter Card has to be installed in the PC.

Downloading of data to individual scale one by one.

SPEC setting on scale :

SPEC 49	CLIENT / SERVER INTERFACE

0	No interface 				3	Not used

1	Ethernet - Coaxial cable		4	4 lines, RS485 (For future use)

2	Ethernet - Twisted cable

SPEC 50	CLIENT / SERVER

0	Client					2	Back-up server

1	Server / Workstation

SPEC 135	PORT NUMBER

* Enter number from 1 to 255

NOTE : Please clear the memory ( [REZERO]+[FEED] when power on ) of the scale

after setting the SPEC for the Ethernet Server / PC connection.

PC & SM-80SX / SM-90 COMMUNICATION VIA TWSWTCP.EXE

PC ETHERNET ADAPTER CARD

Start  Settings  Control Panel  Network  TCP / IP ( Select Properties )

Configure the IP Address and Submask of the PC ethernet adapter card as follows :

IP Address : 192.168.0.XXX where XXX can be any number from 0 to 255

Submask : 255.255.255.0

HOSTS

Text file which contains the IP Addresses of all the SM-80SX / SM-90 scales which is connected to the scale network.

WINDOWS 95 / 98 Operating System : HOSTS is located in WINDOWS folder

WINNT Operating System : HOSTS is located in Winnt\System32\Drivers\Etc folder

Sample of HOSTS :

192.168.0.1	S0001

192.168.0.2	S0002

192.168.0.3	S0003

SCALE PORT NUMBER

TWSWTCP.EXE driver uses the following scale port number via WinSocks :

2000  +  (Port number specified in Spec 135)

Example :  Scale IP Address = 192.168.0.3

Scale Port Number = 2003

##### SM-90 FILES SUPPORTED BY TWSWTCP.EXE

The list of SM-90 files supported by TWSWTCP.EXE is as follows:

UPLOAD FILES ( PC  SM-80SX / SM-90 )

| FILE NAME | COMMAND | UPLOAD DATA FILE |
| --- | --- | --- |
| Department | # TWSWTCP F20.DAT 1 | SM001F20.DAT |
| Main Group | TWSWTCP F23.DAT 1 | SM001F23.DAT |
| PLU | TWSWTCP F25.DAT 1 | SM001F25.DAT |
| Print Format | TWSWTCP F34.DAT 1 | SM001F34.DAT |
| Text | TWSWTCP F38.DAT 1 | SM001F38.DAT |
| Scroll Message | TWSWTCP F39.DAT 1 | SM001F39.DAT |
| Scroll Sequence | TWSWTCP F40.DAT 1 | SM001F40.DAT |
| Preset Key | TWSWTCP F41.DAT 1 | SM001F41.DAT |
| Ingredient | TWSWTCP F50.DAT 1 | SM001F50.DAT |
| Special Message | TWSWTCP F51.DAT 1 | SM001F51.DAT |
| Shop Name | TWSWTCP F52.DAT 1 | SM001F52.DAT |

Note: It is necessary to create the UPLOAD DATA FILE for each individual scale before using the TWSWTCP.EXE driver to upload the respective UPLOAD DATA FILES to the scales.

DELETE FILES ( PC  SM-80SX / SM-90 )

| FILE NAME | COMMAND | DELETE DATA FILE |
| --- | --- | --- |
| Department | # TWSWTCP F90.DAT 1 | SM001F90.DAT |
| Main Group | TWSWTCP F93.DAT 1 | SM001F93.DAT |
| PLU ( Delete Selected Items ) | TWSWTCP F95.DAT 1 | SM001F95.DAT |
| PLU ( Delete All Items ) | TWSWTCP F88.DAT 1 |  |
| Print Format | TWSWTCP F89.DAT 1 | SM001F89.DAT |
| Text | TWSWTCP F98.DAT 1 | SM001F98.DAT |
| Scroll Message | TWSWTCP F99.DAT 1 | SM001F99.DAT |
| Scroll Sequence | TWSWTCP F97.DAT 1 | SM001F97.DAT |
| Preset Key | TWSWTCP F94.DAT 1 | SM001F94.DAT |
| Ingredient | TWSWTCP F96.DAT 1 | SM001F96.DAT |
| Special Message | TWSWTCP F91.DAT 1 | SM001F91.DAT |
| Shop Name | TWSWTCP F92.DAT 1 | SM001F92.DAT |

Note: It is necessary to create the DELETE DATA FILE for each individual scale before using the TWSWTCP.EXE driver to upload the respective DELETE DATA FILES to the scales. However, this is not necessary if this command TWSWTCP F88.DAT 1 is used to delete all PLU items in the scale.

DOWNLOAD FILES ( PC  SM-80SX / SM-90 )

| FILE NAME | COMMAND | DOWNLOAD DATA FILE |
| --- | --- | --- |
| # Department | # TWSWTCP F80.DAT 1 | SM001T80.DAT |
| # Department Total | # TWSWTCP F21.DAT 1 | SM001T21.DAT |
| Main Group | TWSWTCP F83.DAT 1 | SM001T83.DAT |
| Main Group Total | TWSWTCP F24.DAT 1 | SM001T24.DAT |
| PLU | TWSWTCP F85.DAT 1 | SM001T85.DAT |
| PLU Total | TWSWTCP F26.DAT 1 | SM001T26.DAT |
| Store Total | TWSWTCP F46.DAT 1 | SM001T46.DAT |
| Print Format | TWSWTCP F74.DAT 1 | SM001T74.DAT |
| Text | TWSWTCP F78.DAT 1 | SM001T78.DAT |
| Scroll Message | TWSWTCP F79.DAT 1 | SM001T79.DAT |
| Scroll Sequence | TWSWTCP F70.DAT 1 | SM001T70.DAT |
| Preset Key | TWSWTCP F71.DAT 1 | SM001T71.DAT |
| Ingredient | TWSWTCP F60.DAT 1 | SM001T60.DAT |
| Special Message | TWSWTCP F61.DAT 1 | SM001T61.DAT |
| Shop Name | TWSWTCP F62.DAT 1 | SM001T62.DAT |

Note: It is NOT necessary to create the DOWNLOAD DATA FILE for each individual scale. The TWSWTCP.EXE driver will create the DOWNLOAD DATA FILE automatically during the DOWNLOAD operation. The driver will download the respective data files from each individual scale and store them in their respective DOWNLOAD DATA FILES.

##### ERROR CODES GENERATED BY TWSWTCP.EXE

The description of TWSWTCP Scale Driver Error Codes is stored in the RETVALS file and it is located in the same directory as the TWSWTCP scale driver. The list of Error Codes generated by TWSWTCP scale driver is as follows:

RETVALS FILE

| ERROR CODE | SYMBOL | DESCRIPTION |
| --- | --- | --- |
| -1 | # OPEN_FILE_ERR | Error opening input or output file |
| -2 | READ_FILE_ERR | Error reading input file |
| -3 | WRIT_FILE_ERR | Error writing to input or output file |
| -5 | NETWORK_OPEN_ERR | Error connecting to scale |
| -6 | NETWORK_READ_ERR | Error receiving data from scale |
| -7 | NETWORK_WRIT_ERR | Error sending data to scale |
| -8 | MACHINE_READ_ERR | Read error returned by scale |
| -9 | MACHINE_WRIT_ERR | Write error returned by scale |
| -10 | MACHINE_NOREC_ERR | No record error returned by scale |
| -11 | MACHINE_SPACE_ERR | No space error returned by scale |
| -12 | MACHINE_UNDEF_ERR | Undefined error returned by scale |

UPLOAD PLU ITEM FILE VIA TWSWTCP.EXE

The command to upload PLU Item file to scale with IP address 192.168.0.1 is as follows:

TWSWTCP F25.DAT 1

TWSWTCP driver will generate a Scale Number log file called ERROR after uploading. If an error occurs during uploading, an Error Code log file called RETVALS is also generated.

If ERROR file displays 0 means no communication error.

RETVALS log file will not be generated.

If ERROR file displays 1 means unable to upload PLU Item file to scale.

RETVALS file will be generated which shows the error code.

If RETVALS file displays –5 means unable to connect to scale.

It is necessary to create one PLU Item file (SM001F25. DAT).

The command to upload PLU Item file to two scales with IP addresses 192.168.0.2 and 192.168.0.3 is as follows :

TWSWTCP F25.DAT 2 3

If ERROR file displays 2 means unable to upload PLU Item file to one scale only.

RETVALS file will be generated which shows the error code.

If RETVALS file displays –5 means unable to connect to one scale only.

If ERROR file displays 2;3 means unable to upload PLU Item file to both scales.

RETVALS file will be generated which shows the error codes.

If RETVALS file displays –5;–6 means unable to connect to scale with IP address 192.168.0.2 and unable to receive data from scale with IP address 192.168.0.3

It is necessary to create two PLU Item files (SM002F25.DAT & SM003F25.DAT).

Note: Please refer to RETVALS FILE table on page 6 for the list of Error Codes generated by TWSWTCP scale driver.

SM001F25.DAT ( STANDARD )

00000123	0029	0800	0D2004	00000180 11	05	20001230000000

0003	07 0B 52 6F 61 73 74 65 64 20 50 69 67 0C

00

00000456003009008D2104000029501105200045600000000005000301070F4272756E2042616E616E61205069650C00

Sample PLU Item file of SM-80SX / SM-90 scale with IP address 192.168.0.1 for a single weighed PLU item and a single non-weighed PLU item. The sample PLU Item file consists of the following information for both PLU items:

Weighed PLU		Non-weighed PLU

-----------------		-----------------------

- PLU NUMBER			CHAR : 001 – 008	CHAR : 083 - 090

- PLU RECORD SIZE		CHAR : 009 – 012	CHAR : 091 - 094

- PLU STATUS			CHAR : 013 – 016	CHAR : 095 - 098

- PLU STATUS 2			CHAR : 017 – 022	CHAR : 099 - 104

- UNIT PRICE			CHAR : 023 – 030	CHAR : 105 - 112

- LABEL 1 FORMAT		CHAR : 031 – 032	CHAR : 113 - 114

- BARCODE FORMAT		CHAR : 033 – 034	CHAR : 115 - 116

- EAN DATA			CHAR : 035 – 048	CHAR : 117 - 130

- USED BY DATE			CHAR : 049 – 052	CHAR : 131 - 134

- QUANTITY						CHAR : 135 – 138

- QUANTITY SYMBOL					CHAR : 139 - 140

- COMMODITY NAME		CHAR : 053 – 080	CHAR : 141 - 176

- BCC				CHAR : 081 – 082	CHAR : 177 - 178

SM-80SX / SM-90 scale with IP address 192.168.0.1 will display the following information for the Weighed PLU item:

- PLU NUMBER			123

- PLU STATUS			WEIGH

- UNIT PRICE			$1.80

- LABEL 1 FORMAT		‘F1’ FREE FORMAT

- BARCODE FORMAT		FF  CCCCC  XXXXX  CD

- EAN DATA			FLAG : 20  &  ITEM CODE : 00123

- USED BY DATE			3 DAYS

- COMMODITY CHAR SIZE	M3

- COMMODITY NAME		Roasted Pig

SM-80SX / SM-90 scale with IP address 192.168.0.1 will also display the following information for the Non-weighed PLU item:

- PLU NUMBER			456

- PLU STATUS			NON-WEIGH

- UNIT PRICE			$29.50

- LABEL 1 FORMAT		‘F1’ FREE FORMAT

- BARCODE FORMAT		FF  CCCCC  XXXXX  CD

- EAN DATA			FLAG : 20  &  ITEM CODE : 00456

- USED BY DATE			5 DAYS

- QUANTITY			3

- QUANTITY SYMBOL TYPE	PCS

- COMMODITY CHAR SIZE	M3

- COMMODITY NAME		Brun Banana Pie

UPLOAD PLU DELETE FILE VIA TWSWTCP.EXE

The command to upload PLU Delete file to scale with IP address 192.168.0.1 is as follows :

TWSWTCP F95.DAT 1

TWSWTCP driver will generate a log file called ERROR after communication. If an error occurs during uploading, an Error Code log file called RETVALS is also generated.

If ERROR file displays 0 means no communication error.

If ERROR file displays 1 means unable to upload PLU Delete file to scale.

It is necessary to create one PLU Delete file (SM001F95.DAT).

The command to upload PLU Delete file to two scales with IP addresses 192.168.0.2 and 192.168.0.3 is as follows :

TWSWTCP F95.DAT 2 3

If ERROR file displays 2 means unable to upload PLU Delete file to one scale only.

If ERROR file displays 2;3 means unable to upload PLU Delete file to both scales.

It is necessary to create two PLU Delete files (SM002F95.DAT & SM003F95.DAT).

Note: Please refer to RETVALS FILE table on page 6 for the list of Error Codes generated by TWSWTCP scale driver.

SM001F95.DAT ( STANDARD )

000000030000002200000111

Sample PLU Delete file of SM-80SX / SM-90 scale with IP address 192.168.0.1 for three  PLU items. The sample PLU Delete file consists of the following information :

- PLU NUMBER 3			CHAR : 001 - 008

- PLU NUMBER 22		CHAR : 009 - 016

- PLU NUMBER 111		CHAR : 017 - 024

SM-80SX / SM-90 scale with IP address 192.168.0.1 will delete the following PLU items :

- PLU NUMBER			3

- PLU NUMBER			22

- PLU NUMBER			111

Note: The following command is used to delete ALL PLU items in the scale without creating a PLU Delete file.

TWSWTCP F88.DAT 1

DOWNLOAD PLU ITEM FILE VIA TWSWTCP.EXE

The command to download PLU Item file from scale with IP address 192.168.0.1 is as follows :

TWSWTCP F85.DAT 1

TWSWTCP driver will generate a log file called ERROR after communication. If an error occurs during uploading, an Error Code log file called RETVALS is also generated.

If ERROR file displays 0 means no communication error.

If ERROR file displays 1 means unable to download PLU Item file from scale.

The command to download PLU Item file from two scales with IP addresses 192.168.0.2 and 192.168.0.3 is as follows :

TWSWTCP F85.DAT 2 3

If ERROR file displays 2 means unable to download PLU Item file from one scale only.

If ERROR file displays 2;3 means unable to download PLU Item file from both scales.

Note: It is NOT necessary to create any PLU Item upload file SM001F85.DAT for any   SM-80SX / SM-90 scale. The sample PLU Item download file SM001T85.DAT consists of PLU Item information for all PLU items in the SM-80SX / SM-90 scale with IP address 192.168.0.1

Note: Please refer to RETVALS FILE table on page 6 for the list of Error Codes generated by TWSWTCP scale driver.

SM001T85.DAT ( STANDARD )

00000123002908000D2004000001801105200012300000000003070B526F6173746564205069670C0000000456003009008D2104000029501105200045600000000005000301070F4272756E2042616E616E61205069650C00009999980013740010000000000000FF099881009999990013750010000000000000FF099980E2

# The SM001T85.DAT download item file from scale has the same information as the SM001F25.DAT upload item file to scale except that the last byte of data E2 represents the

# End-of-Text. Furthermore, it consists of information for default items PLU 999998 and

# PLU 999999 too.

DOWNLOAD PLU TOTAL FILE VIA TWSWTCP.EXE

The command to download PLU Total file from scale with IP address 192.168.0.1 is as follows :

TWSWTCP F26.DAT 1

TWSWTCP driver will generate a log file called ERROR after communication. If an error occurs during uploading, an Error Code log file called RETVALS is also generated.

If ERROR file displays 0 means no communication error.

If ERROR file displays 1 means unable to download PLU Total file from scale.

The command to download PLU Total file from two scales with IP addresses 192.168.0.2 and 192.168.0.3 is as follows :

TWSWTCP F26.DAT 2 3

If ERROR file displays 2 means unable to download PLU Total file from one scale only.

If ERROR file displays 2;3 means unable to download PLU Total file from both scales.

Note: It is NOT necessary to create any PLU Total upload file SM001F26.DAT for any  SM-80SX / SM-90 scale. The sample PLU Total download file SM001T26.DAT consists of PLU Transaction information for those PLU items with sales transaction in the SM-80SX / SM-90 scale with IP address 192.168.0.1

Note: Please refer to RETVALS FILE table on page 6 for the list of Error Codes generated by TWSWTCP scale driver.

SM001T26.DAT ( STANDARD )

# 00000123006D20001230000000000997000001100000011000010000092600000110000001100000011000010000092600000110000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000240898

# 00000456006D20004560000000000997000008200000082000020000180600000820000008200000082000020000180600000820000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000240898E2

Sample PLU Total download file from SM-80SX / SM-90 scale with IP address 192.168.0.1 for two PLU items. The sample PLU Total download file consists of the following information :

PLU 123	PLU 456	End-of-Text

- PLU NUMBER			CHAR : 001 - 008	219 - 226 	437 - 438

- PLU TOTAL RECORD SIZE	CHAR : 009 - 012	227 - 230

- EAN CODE			CHAR : 013 - 026	231 - 244

- DUMMY				CHAR : 027 - 028	245 - 246

- MAIN GROUP CODE		CHAR : 029 - 032	247 – 250

DAILY TOTAL

- TOTAL ACTUAL PRICE	CHAR : 033 - 040	251 - 258

- TOTAL PLANNED PRICE	CHAR : 041 - 048	259 - 266

- TOTAL PCS			CHAR : 049 - 052	267 - 270

- TOTAL WEIGHT			CHAR : 053 - 060	271 - 278

- TOTAL PROFIT			CHAR : 061 - 068	279 – 286

TERM TOTAL

- TOTAL ACTUAL PRICE	CHAR : 069 - 076	287 - 294

- TOTAL PLANNED PRICE	CHAR : 077 - 084	295 - 302

- TOTAL PCS			CHAR : 085 - 088	303 - 306

- TOTAL WEIGHT			CHAR : 089 - 096	307 - 314

- TOTAL PROFIT			CHAR : 097 - 104	315 – 322

PREPACK DAILY TOTAL

- TOTAL ACTUAL PRICE	CHAR : 105 - 112	323 - 330

- TOTAL PLANNED PRICE	CHAR : 113 - 120	331 - 338

- TOTAL PCS			CHAR : 121 - 124	339 - 342

- TOTAL WEIGHT			CHAR : 125 - 132	343 - 350

- TOTAL PROFIT			CHAR : 133 - 140	351 – 358

PREPACK TERM TOTAL

- TOTAL ACTUAL PRICE	CHAR : 141 - 148	359 - 366

- TOTAL PLANNED PRICE	CHAR : 149 - 156	367 - 374

- TOTAL PCS			CHAR : 157 - 160	375 - 378

- TOTAL WEIGHT			CHAR : 161 - 168	379 - 386

- TOTAL PROFIT			CHAR : 169 - 176	387 – 394

DISCOUNT TOTAL

- TOTAL ACTUAL PRICE	CHAR : 177 - 184	395 - 402

- TOTAL PLANNED PRICE	CHAR : 185 - 192	403 - 410

- TOTAL PCS			CHAR : 193 - 196	411 - 414

- TOTAL WEIGHT			CHAR : 197 - 204	415 - 422

- TOTAL PROFIT			CHAR : 205 - 212	423 – 430

- LAST ACT DATE		CHAR : 213 - 218	431 - 436

# Note: The last byte of data E2 represents the End-of-Text.

UPLOAD PRESET KEY ASSIGNMENT FILE VIA TWSWTCP.EXE

The command to upload Preset Key Assignment file to scale with IP address 192.168.0.1 is as follows :

TWSWTCP F41.DAT 1

TWSWTCP driver will generate a log file called ERROR after communication. If an error occurs during uploading, an Error Code log file called RETVALS is also generated.

If ERROR file displays 0 means no communication error.

If ERROR file displays 1 means unable to upload PLU data file to scale.

It is necessary to create one Preset Key Assignment file (SM001F41.DAT).

The command to upload PLU data file to two scales with IP addresses 192.168.0.2 and 192.168.0.3 is as follows :

TWSWTCP F41.DAT 2 3

If ERROR file displays 2 means unable to upload PLU data file to one scale only.

If ERROR file displays 2;3 means unable to upload PLU data file to both scales.

Necessary to create two Preset Key Assignment files (SM002F41.DAT & SM003F41.DAT).

Note: Please refer to RETVALS FILE table on page 6 for the list of Error Codes generated by TWSWTCP scale driver.

SM001F41.DAT ( STANDARD )

0000000100130000012300504C5520313233200000000800130000045600504C552034353620

Sample Preset Key Assignment file of SM-80SX / SM-90 scale with IP address 192.168.0.1 for two PLU preset keys. The sample Preset Key Assignment file consists of the following information for both PLU preset keys:

Preset Key # 1		Preset Key # 8

-----------------		-----------------------

- PRESET NUMBER		CHAR : 001 – 008	CHAR : 039 - 046

- PRESET RECORD SIZE		CHAR : 009 – 012	CHAR : 047 - 050

- PRESET KEY SWITCH		CHAR : 013 – 020	CHAR : 051 - 058

- PRESET STATUS		CHAR : 021 – 022	CHAR : 059 - 060

- PRESET NAME			CHAR : 023 – 038	CHAR : 061 - 076

SM-80SX / SM-90 scale with IP address 192.168.0.1 will display the following information for both Preset Keys:

- PRESET KEY NUMBER		1

- PLU NUMBER			123

- PRESET KEY NAME		PLU 123

- PRESET KEY NUMBER		8

- PLU NUMBER			456

- PRESET KEY NAME		PLU 456

SM-80SX / SM-90 FILE STRUCTURE

- CODE TYPE ( 1 Byte )

- HONG KONG & MALAYSIA

| - LANGUAGE CODE | - VALUE (HEX) |
| --- | --- |
| - GB | - 00H |
| - BIG 5 | - 01H |
| - JIS | - 02H |
| - ASCII | - 03H |
| - PRESET KEY | - 04H |

- JAPAN & TAIWAN

| - LANGUAGE CODE | - VALUE (HEX) |
| --- | --- |
| - TERAOKA | - 00H |
| - JIS | - 01H |
| - ASCII | - 02H |
| - BIG 5 | - 03H |

- STANDARD

| - LANGUAGE CODE | - VALUE (HEX) |
| --- | --- |
| - ASCII | - 00H |
| - TERAOKA | - 01H |
| - PRESET KEY | - 02H |

- Note : CODE TYPE ( 1 Byte ) is used to determine the language code for the characters.

- Applicable to DEPARTMENT ( 20H ) , MAIN GROUP ( 23H ) , CLERK ( 44H )

- & SCROLL MESSAGE ( 3FH ) files only.

- CODE TYPE is always set to 00H for CHINA & KOREA country specification.

- Use Preset Key 40 (Pole Type) or Preset Key 15 (Bench Type) to change CODE

- TYPE on SM-80SX / SM-90 scale. However, use Preset Key 24 (Bench Type) if

- the scale is set to HONG KONG or MALAYSIA country specification.

- DEPARTMENT FILE - (20H)

- DEPARTMENT CODE				-	4 BYTES	(BCD)

- DEPARTMENT RECORD SIZE			-	2 BYTES	(HEX)

- DEPARTMENT NAME				-	16 BYTES	(ASCII)

- CODE TYPE						-	1 BYTE	(HEX)

- Maximum number of departments = 99

- No. of bytes occupy = 99 × 23 = 2277 BYTES

- DEPARTMENT TOTAL FILE - (21H)

- DEPARTMENT CODE				-	4 BYTES	(BCD)

- DEPARTMENT RECORD SIZE			-	2 BYTES	(HEX)

- TOTAL DAILY					-	24 BYTES

- TOTAL CUSTOMER			-	4 BYTES

- TOTAL PCS				-	4 BYTES

- TOTAL ACTUAL PRICE		-	4 BYTES

- TOTAL PLANNED PRICE		-	4 BYTES

- TOTAL WEIGHT			-	4 BYTES

- TOTAL PROFIT			-	4 BYTES

- TOTAL MONTHLY					-	24 BYTES

- TOTAL CUSTOMER			-	4 BYTES

- TOTAL PCS				-	4 BYTES

- TOTAL ACTUAL PRICE		-	4 BYTES

- TOTAL PLANNED PRICE		-	4 BYTES

- TOTAL WEIGHT			-	4 BYTES

- TOTAL PROFIT			-	4 BYTES

- TOTAL TERM					-	24 BYTES

- TOTAL CUSTOMER			-	4 BYTES

- TOTAL PCS				-	4 BYTES

- TOTAL ACTUAL PRICE		-	4 BYTES

- TOTAL PLANNED PRICE		-	4 BYTES

- TOTAL WEIGHT			-	4 BYTES

- TOTAL PROFIT			-	4 BYTES

- TOTAL LAST MONTH				-	24 BYTES

- TOTAL CUSTOMER			-	4 BYTES

- TOTAL PCS				-	4 BYTES

- TOTAL ACTUAL PRICE		-	4 BYTES

- TOTAL PLANNED PRICE		-	4 BYTES

- TOTAL WEIGHT			-	4 BYTES

- TOTAL PROFIT			-	4 BYTES

- DEPARTMENT LAST ACT DATE	-	3 BYTES (6 digits BCD, DD.MM.YY)

- Maximum number of departments = 99

- No. of bytes occupy = 99 × (9 + 24 × 4) = 10395 Bytes

- MAIN GROUP FILE - (23H)

- MAIN GROUP CODE				-	4 BYTES	(BCD)

- MAIN GROUP RECORD SIZE			-	2 BYTES	(HEX)

- LINKED DEPARTMENT CODE			-	1 BYTE	(BCD)

- MAIN GROUP NAME				-	16 BYTES	(ASCII)

- CODE TYPE						-	1 BYTE	(HEX)

- TAX RATE REF #					-	1 BYTE	(BCD)

- Maximum number of main groups = 999

- No. of bytes occupy = 999 × 25 = 24975 BYTES

- MAIN GROUP TOTAL FILE - (24H)

- MAIN GROUP CODE				-	4 BYTES	(BCD)

- MAIN GROUP RECORD SIZE			-	2 BYTES	(HEX)

- LINKED DEPARTMENT CODE			-	1 BYTE	(BCD)

- DUMMY						-	1 BYTE

- TOTAL DAILY					-	36 BYTES

- TOTAL CUSTOMER			-	4 BYTES

- TOTAL PCS				-	4 BYTES

- TOTAL ACTUAL PRICE		-	4 BYTES

- TOTAL PLANNED PRICE		-	4 BYTES

- TOTAL WEIGHT			-	4 BYTES

- TOTAL DISCOUNT PRICE		-	4 BYTES

- TOTAL NO. OF REFUND		-	4 BYTES

- TOTAL REFUND PRICE		-	4 BYTES

- TOTAL PROFIT			-	4 BYTES

- TOTAL TERM					-	36 BYTES

- TOTAL CUSTOMER			-	4 BYTES

- TOTAL PCS				-	4 BYTES

- TOTAL ACTUAL PRICE		-	4 BYTES

- TOTAL PLANNED PRICE		-	4 BYTES

- TOTAL WEIGHT			-	4 BYTES

- TOTAL DISCOUNT PRICE		-	4 BYTES

- TOTAL NO. OF REFUND		-	4 BYTES

- TOTAL REFUND PRICE		-	4 BYTES

- TOTAL PROFIT			-	4 BYTES

- MAIN GROUP LAST ACT DATE	-	3 BYTES (6 digits BCD, DD.MM.YY)

- Maximum number of main groups = 999

- No. of bytes occupy = 999 × (11 + 36 × 2) = 82917 BYTES

1.   PLU FILE - (25H)

- PLU NUMBER				-	4 BYTES (BCD)

- PLU RECORD SIZE			-	2 BYTES (HEX)

- PLU STATUS				-	2 BYTES (BIN)

- PLU STATUS 2			-	3 BYTES (BIN)

- UNIT PRICE				-	4 BYTES (BCD)

- LABEL 1 FORMAT			-	1 BYTE (HEX)

- LABEL 2 FORMAT			-	1 BYTE (HEX)

- BARCODE FORMAT			-	1 BYTE (HEX)

- EAN DATA				-	7 BYTES (BCD)

- MAIN GROUP CODE			-	2 BYTES (BCD)

- SELL BY DATE			-	2 BYTES (3 digits BCD, -no. of days)

- SELL BY TIME			-	2 BYTES (4 digits BCD, HH:MM)

- USED BY DATE			-	2 BYTES (3 digits BCD, -no. of days)

- PACKED DATE			-	2 BYTES (3 digits BCD, -no. of days)

- PACKED TIME			-	2 BYTES (4 digits BCD, HH:MM)

- COST					-	4 BYTES (BCD)

- PLU TARE				-	2 BYTES (BCD)

- QUANTITY				-	2 BYTES (BCD)

- QUANTITY SYMBOL TYPE		-	1 BYTE (BCD)

- SPECIAL MESSAGE #			-	1 BYTE (BCD)

- INGREDIENT #			-	1 BYTE (BCD)

- PLACE NUMBER			-	1 BYTE (BCD)

- IMAGE1				-	1 BYTE (BCD)

- IMAGE2				-	1 BYTE (BCD)

- IMAGE3				-	1 BYTE (BCD)

- IMAGE4				-	1 BYTE (BCD)

- IMAGE5				-	1 BYTE (BCD)

- IMAGE6				-	1 BYTE (BCD)

- IMAGE7				-	1 BYTE (BCD)

- IMAGE8				-	1 BYTE (BCD)

- IMAGE9				-	1 BYTE (BCD)

- IMAGE10				-	1 BYTE (BCD)

*PLU STATUSB				-	4 BYTES	(BIN)

*PLU STATUS 2B			-	8 BYTES	(BIN)

*2ND PRICE				-	4 BYTES	(BCD) (DP90)

*2ND QUANTITY			-	2 BYTES	(BCD) (DP90)

*C_PLU_PRESET			-	33 BYTES	(ASCII) (DP90)

*BONUS POINT				-	2 BYTES	(BCD)

*REFERENCE PLU NUMBER		-	4 BYTES	(BCD)

COUPLED PLU NO			- 	4 BYTES 	(BCD) (VER XX.14)

- DISCOUNT				-	30 BYTES

- NUTRITION				-	80 BYTES (ONLY IN U1)

DISCOUNT DAY OF WEEK		-	1 BYTE	(BIN) (VER XX.14)

- COMMODITY				-	1 - 412 BYTES (ASCII)

- INGREDIENT				-	1 - 1545 BYTES (ASCII)

- SPECICAL MESSAGE			-	1 - 824 BYTES (ASCII)

- BCC					-	1 BYTE (2 digits BCD, 00)

No. of bytes occupy = VARIABLE DEPENDENT ON PLU STATUS 2 & PLU STATUS 2B

Note:	1.  With effect from SM90 version 2.26, INGREDIENT for U1 = (1545 + 8652 bytes)

2.  REFERENCE PLU NUMBER added from SM90 VER 2.62 & above.

- 3.  Fields denoted with * are modified to cater for unified database.

- PLU STATUS (2 Bytes) : BYTE 1 followed by BYTE 2

| - BYTE 1 | - BIT 0 | - UNIT: | - 0 - 1 | - - - - | - Weighing - Non-weighing |
| --- | --- | --- | --- | --- | --- |
|  | - BIT 1 | - UNIT PRICE: | - 0 - 1 | - - - - | - 1ST Price - 2ND Price |
|  | - BIT 2 | - SELL BY DATE: | - 0 - 1 | - - - - | - INHIBIT - ALLOW |
|  | - BIT 3 | - USED BY DATE: | - 0 - 1 | - - - - | - INHIBIT - ALLOW |
|  | - BIT 4 | - PACKED DATE: | - 0 - 1 | - - - - | - INHIBIT - ALLOW |
|  | - BIT 5 | - SELL BY TIME: | - 0 - 1 | - - - - | - INHIBIT - ALLOW |
|  | - BIT 6 | - PACKED TIME: | - 0 - 1 | - - - - | - INHIBIT - ALLOW |
|  | - BIT 7 | - RTC PACKED TIME: | - 0 - 1 | - - - - | - RTC - KEY ENTRY |
| - BYTE 2 | - BIT 0 | - RTC SELL BY TIME: | - 0 - 1 | - - - - | - RTC - KEY ENTRY |
|  | - BIT 1 | - PRICE BASED PER UNIT: | - 0 - 1 | - - - - | - Price base per KG - Price base per 100G |
|  | - BIT 3 | - NUTRIITION PRINT: | - 0 - 1 | - - - - | - INHIBIT - ALLOW |
|  | - BIT 4 | - UNIT PRICE OVERRIDE: | - 0 - 1 | - - - - | - INHIBIT - ALLOW |
|  | # BIT 5 | SELL BY DATE SOURCE: | 0 1 | - - | RTC DATE PACKED DATE |
|  | BIT 6 | SECURITY TAG ISSUE CONDITION: | 0 1 | - - | THRESHOLD THIS PLU |
|  | BIT 7 | NEGATIVE PLU (VER XX.14) | 0 1 | - - | INHIBIT ALLOW |

Note:

- NUTRITION PRINT bit is for YES or NO when prompted for printing of

nutrition in PLU programming.

- ** If SM90 SPEC 612 is set as “500G / 1 KG”, setting this bit 1 determines price

- base as per 500 G and with effect from VER 2.69 & onwards.

- Non-applicable to TU  & UK Country Code.

- Sell by date source (BIT 5 of PLU STATUS BYTE 2) currently used by DP90 &

- touch screen version of SM90 only.

- Security tag issue condition (BIT 6 of PLU STATUS BYTE 2) with effect from

- SM90 VER 2.79 onwards.

PLU STATUS 2 (3 Bytes) : BYTE 1 followed by BYTE 2 and BYTE 3

| - BYTE 1 | - BIT 0 | - LABEL 1 FORMAT: | - 0 - 1 | - - - - | - DEFAULT - INDIVIDUAL |
| --- | --- | --- | --- | --- | --- |
|  | - BIT 1 | - LABEL 2 FORMAT: | - 0 - 1 | - - - - | - INHIBIT - ALLOW |
|  | - BIT 2 | - BARCODE FORMAT: | - 0 - 1 | - - - - | - DEFAULT - INDIVIDUAL |
|  | - BIT 3 | - ITEM CODE: | - 0 - 1 | - - - - | - INHIBIT - ALLOW |
|  | - BIT 4 | - MAIN GROUP CODE: | - 0 - 1 | - - - - | - MG CODE = 997 - MG CODE  997 |
|  | - BIT 5 | - COST: | - 0 - 1 | - - - - | - INHIBIT - ALLOW |
|  | - BIT 6 | - PLU TARE: | - 0 - 1 | - - - - | - INHIBIT - ALLOW |
|  | - BIT 7 | - QUANTITY: | - 0 - 1 | - - - - | - INHIBIT - ALLOW |
| - BYTE 2 | - BIT 0 | - QUANTITY SYMBOL: | - 0 - 1 | - - - - | - INHIBIT - ALLOW |
|  | - BIT 1 | - SPECIAL MESSAGE #: | - 0 - 1 | - - - - | - INHIBIT - ALLOW |
|  | - BIT 2 | - INGREDIENT #: | - 0 - 1 | - - - - | - INHIBIT - ALLOW |
|  | - BIT 3 | - DISCOUNT: | - 0 - 1 | - - - - | - INHIBIT - ALLOW |
|  | - BIT 4 | - NUTRITION | - 0 - 1 | - - - - | - NOT U1 - U1 |
|  | - BIT 5 | - COMMODITY | - 0 - 1 | - - - - | - INHIBIT - ALLOW |
|  | - BIT 6 | - SPECIAL MESSAGE: | - 0 - 1 | - - - - | - INHIBIT - ALLOW |
|  | - BIT 7 | - INGREDIENT: | - 0 - 1 | - - - - | - INHIBIT - ALLOW |
| - BYTE 3 | - BIT 0 | - SELL BY DATE: | - 0 - 1 | - - - - | - INHIBIT - ALLOW |
|  | - BIT 1 | - SELL BY TIME: | - 0 - 1 | - - - - | - INHIBIT - ALLOW |
|  | - BIT 2 | - USED BY DATE: | - 0 - 1 | - - - - | - INHIBIT - ALLOW |
|  | - BIT 3 | - PACKED DATE: | - 0 - 1 | - - - - | - INHIBIT - ALLOW |
|  | - BIT 4 | - PACKED TIME: | - 0 - 1 | - - - - | - INHIBIT - ALLOW |
|  | - BIT 5 | - PLACE NUMBER | - 0 - 1 | - - - - | - INHIBIT - ALLOW |
|  | - BIT 6 | - IMAGE NUMBER | - 0 - 1 | - - - - | - INHIBIT - ALLOW |
|  | *BIT 7 | PLU STATUSB & PLU STATUS 2B | 0 1 | - - | INHIBIT ALLOW |

PLU STATUS 2B (8 Bytes) : BYTE 1

| BYTE 1 | *BIT 0 | 2ND PRICE (DP90): | 0 1 | - - | INHIBIT ALLOW |
| --- | --- | --- | --- | --- | --- |
|  | *BIT 1 | 2ND QUANTITY (DP90): | 0 1 | - - | INHIBIT ALLOW |
|  | *BIT 2 | PRESET NAME (DP90): | 0 1 | - - | INHIBIT ALLOW |
|  | *BIT 3 | # BONUS POINT | 0 1 | - - | INHIBIT ALLOW |
|  | *BIT 4 | REFERENCE PLU NUMBER | 0 1 | - - | INHIBIT ALLOW |
|  | BIT 5 | DISCOUNT DAY OF THE WEEK (VER XX.14) | 0 1 | - - | INHIBIT ALLOW |
|  | BIT 6 | COUPLED PLU (VER XX.14) | 0 1 | - - | INHIBIT ALLOW |

Note:	Bits denoted with * are modified to cater for unified database.

LABEL FORMAT

All countries except U1 Country Code

| - FORMAT # | - VALUE (HEX) |
| --- | --- |
| - DEFAULT | - 00H |
| - T1 – T12 | - 01H – 0CH |
| - S | - 0DH |
| - A | - 0EH |
| - B | - 0FH |
| - C | - 10H |
| - F1 – F8 | - 11H – 18H |

U1 Country Code Only

| - FORMAT # | - VALUE (HEX) |
| --- | --- |
| DEFAULT | 00H |
| A | 01H |
| B | 02H |
| C | 03H |
| U2 – U8 | 04H – 0AH |
| T5 – T10 | 0B – 10 H |
| F1 – F8 | 11H – 18H |

- EAN BARCODE FORMAT (1 Byte)

| ###### VALUE | EAN DATA ( 7 Bytes ) | DESCRIPTIONS |
| --- | --- | --- |
| 00H | Default | Default (From Specification) |
| 01H | F1F2 CCCCC XCD XXXX CD 0 | F1F2 CCCCC XCD XXXX CD (13 Digit NON-PLU) |
| 02H | 0 F2 CCCCCC XCD XXXX CD | F2 CCCCCC XCD XXXX CD (13 Digit NON-PLU) |
| 03H | F1F2 CCCCC 0 XXXX CD 0 | F1F2 CCCCC 0 XXXX CD (13 Digit NON-PLU) |
| 04H | F1F2 CCCCCC XXXX CD 0 | F1F2 CCCCCC XXXX CD (13 Digit NON-PLU) |
| 05H | F1F2 CCCCC XXXXX CD 0 | F1F2 CCCCC XXXXX CD (13 Digit NON-PLU) |
| 06H | 0 F2 CCCCCC XXXXX CD | F2 CCCCCC XXXXX CD (13 Digit NON-PLU) |
| 07H | 0 F2 CCCCC XXXXXX CD | F2 CCCCC XXXXXX CD (13 Digit NON-PLU) |
| 08H | F1F2 CCCCCCCCCC CD 0 | F1F2 CCCCCCCCCC CD (13 Digit PLU) |
| 09H | F1F2 CCCC XXXXXX CD 0 | F1F2 CCCC XXXXXX CD (13 Digit NON-PLU) |
| 0AH | F1F2 CCCCC CD 000000 | F1F2 CCCCC CD (8 Digit PLU) |
| 0BH | 0 F2 CC XXXX CD 00000 | F2 CC XXXX CD (8 Digit NON-PLU) |
| 0CH | NON BARCODE | NON BARCODE (NON BARCODE) |
| 0DH | F1 0 CCCCC XCD XXXX CD 0 | F1X2 CCCCC XCD XXXX CD (13 Digit NON-PLU) |
| 0EH | F1 0 CCCCCC XXXX CD 0 | F1X2 CCCCCC XXXX CD (13 Digit NON-PLU) |
| 0FH | F1F2 CCCC XCD XXXXX CD 0 | F1F2 CCCC XCD XXXXX CD (13 Digit NON-PLU) |
| 10H | 0 F2 CCCCC XCD XXXXX CD | F2 CCCCC XCD XXXXX CD (13 Digit NON-PLU) |
| 11H | F1F2 CCC XXXXXXX CD 0 | F1F2 CCC XXXXXXX CD (13 Digit NON-PLU) |
| 12H | F1F2 CC XXXXXXXX CD 0 | F1F2 CC XXXXXXXX CD (13 Digit NON-PLU) |
| 13H | PLU NO., WEIGHT, TTL PRICE | CCC WWWW PPPPP CD (13 Digit NON-PLU) |
| 14H | NON BARCODE | NON BARCODE (NON BARCODE) |
| 15H | F1F2 CCCCC PCD XXXX CD 0 | F1F2 CCCCC PCD XXXX CD (13 Digit NON-PLU) |

Note: 	XCD	- center checksum calculated from Xs

PCD	- center checksum calculated from flag data & Cs

Xs 	- can be quantity, price, weight or user programmable depending on SPEC 1

and SPEC 153.

If SPEC 1 ( SM-80SX / SM-90 ) is set to user programmable, the last byte of the EAN DATA (7th byte) determines Xs definition as follows:

FORMAT	LAST BYTE OF EAN DATA	Xs

EAN		00 (HEX)			PRICE

EAN		01 (HEX)			WEIGHT

EAN		02 (HEX)			QUANTITY

- ITF BARCODE FORMAT (1 Byte)

| ###### VALUE | EAN DATA(7 Bytes) | DESCRIPTIONS |
| --- | --- | --- |
| 00H | Default | Default (From Specification) |
| 01H | FF CCCCC 00000 90 | FF CCCCC XXXX WWWW CD (16 Digit NON-PLU) |
| 02H | 0F CCCCCC 0000 90 | F CCCCCC XXXX WWWW CD (16 Digit NON-PLU) |
| 03H | FF CCCCC 00000 90 | 0FF CCCCC 0 XXXX WWWW CD (18 Digit NON-PLU) |
| 04H | FF CCCCCC 0000 90 | 0FF CCCCCC XXXX WWWW CD (18 Digit NON-PLU) |
| 05H | FF CCCCC 00000 90 | FF CCCCC XXXXX WWWWW CD (18 Digit NON-PLU) |
| 06H | 0F CCCCCC 0000 90 | F CCCCCC XXXXX WWWWW CD (18 Digit NON-PLU) |
| 07H | 0F CCCCC 00000 90 | 0F CCCCC XXXXXX WWWWWW CD (20 Digit NON-PLU) |
| 08H | FF CCCCCCCCCC 90 | 0FF CCCCCCCCCC CD (14 Digit PLU) |
| 09H | FF CCCC 000000 90 | 0FF CCCC XXXXXX WWWWWW CD (20 Digit NON-PLU) |
| 0AH | FF CCCCC 00000 90 | FF CCCCC CD (8 Digit PLU) |
| 0BH | 0F CC 00000000 90 | F CC XXXX WWWW CD (12 Digit NON-PLU) |
| 0CH | NON BARCODE | NON BARCODE (NON BARCODE) |
| 0DH | F0 CCCCC 00000 90 | FX CCCCC XXXX WWWW CD (16 Digit NON-PLU) |
| 0EH | F0 CCCCCC 0000 90 | 0FX CCCCCC XXXX WWWW CD (18 Digit NON-PLU) |
| 0FH | FF CCCC 000000 90 | 0FF CCCC XXXXX WWWWW CD (18 Digit NON-PLU) |
| 10H | 0F CCCCC 00000 90 | 0F CCCCC XXXXX WWWWW CD (18 Digit NON-PLU) |
| 11H | FF CCC 0000000 90 | FF CCC XXXXXXX WWWWWWW CD (20 Digit NON-PLU) |
| 12H | FF CC 00000000 90 | 0FF CC XXXXXXXX WWWWWWWW CD (22 Digit NON-PLU) |
| 13H | 000000000000 90 | 0CCC WWWW PPPPP CD (14 Digit NON-PLU) |
| 14H | NON BARCODE | NON BARCODE (NON BARCODE) |
| 15H | FF CCCCC 00000 90 | FF CCCCC XXXX WWWW CD (16 Digit NON-PLU) |

The following points on ITF Barcode Format apply to SM90 version 2.28 & above:

The UPPER 4 bits of the last byte of the EAN DATA is also used to set the Barcode Format as EAN or ITF regardless of SPEC 1 (SM-80SX / SM-90 ). If the UPPER 4 bits is set to 9, the Barcode Format is ITF type. Otherwise, it is EAN type.

Note:	Ws	- will be weight for weigh item, or quantity for non-weigh item

Xs	- can be quantity, price, weight or user programmable depending on SPEC 1

and SPEC 153.

If SPEC 1 ( SM-80SX / SM-90 ) is set to user programmable, the last byte of the EAN DATA (7th byte) determines Xs definition as follows:

FORMAT	LAST BYTE OF EAN DATA	Xs

ITF		90 (HEX)			PRICE

ITF		91 (HEX)			WEIGHT

ITF		92 (HEX)			QUANTITY

- QUANTITY SYMBOL TYPE (1 Byte)

| - Value | - Status |
| --- | --- |
| - 0 | - No symbol |
| - 1 | - Pcs |
| - 2 | - For |
| - 3 | - Kg |
| - 4 | - LB |
| - 5 | - G |
| - 6 | - OZ |
| - 7 | - METRICS (not displayed) |

- DISCOUNT (30 Bytes)

- START DATE FOR DISCOUNT	-	4 BYTES (6 digits BCD, DD.MM.YY)

- START TIME FOR DISCOUNT	-	2 BYTES (4 digits BCD, HH:MM)

- END DATE FOR DISCOUNT	-	4 BYTES (6 digits BCD, DD.MM.YY)

- END DATE FOR DISCOUNT	-	2 BYTES (4 digits BCD, HH:MM)

- FIRST LIMIT FOR DISCOUNT	-	4 BYTES (BCD)

- DISCOUNT WITHIN 1ST LIMIT	-	4 BYTES (BCD)

- SECOND LIMIT FOR DISCOUNT	-	4 BYTES (BCD)

- DISCOUNT WIHTIN 2ND LIMIT	-	4 BYTES (BCD)

- TYPE OF DISCOUNT		-	1 BYTE (BCD)

- TYPE OF MARKDOWN		-	1 BYTE (BCD)

- Type of Discount (1 Byte)

| - Value | - Status |
| --- | --- |
| - 0 | - No discount |
| - 1 | - Free item |
| - 2 | - Unit price discount |
| - 3 | - Unit price percentage discount |
| - 4 | - Total price discount |
| - 5 | - Total price percentage discount |
| - 6 | - Fixed price discount |

- Type of Markdown (1 Byte)

| - Value | - Status |
| --- | --- |
| - 0 | - No markdown |
| - 1 | - Unit price markdown |
| - 2 | - Price markdown |
| - 3 | - Unit price and price markdown |

- COMMODITY NAME , INGREDIENT and SPECIAL MESSAGE

- Every line of commodity name , ingredient and special message is separated from the next line with ‘CR’ (0DH). End of these data is indicated by ‘FF’ (0CH). First Byte of each line represents the Character Size. Second Byte of each line indicates the number of characters for that line.

- Character Size

| - Character | - Data Code (HEX) | - Size | - Country |
| --- | --- | --- | --- |
| - S1 - S2 - S3 - S4 - S5 | - 00 - 01 - 02 - 03 - 04 | - 5 × 7 - 6 × 10 - 7 × 12 - 7 × 16 - 9 × 22 | - STANDARD |
| - M1 - M2 - M3 - M4 - M5 | - 05 - 06 - 07 - 08 - 09 | - 10 × 14 - 12 × 20 - 14 × 24 - 14 × 32 - 18 × 44 | - STANDARD |
| - B1 - B2 - B3 - B4 - B5 - B6 | - 0A - 0B - 1A - 1B - 1C - 1D | - 24 × 24 - 48 × 48 - 24 × 48 - 16 × 15 - 16 × 30 - 32 × 30 | - HONG KONG - & - TAIWAN |
| - J1 - J2 - J3 - J4 | - 10 - 11 - 12 - 13 | - 16 × 16 - 16 × 32 - 32 × 32 - 24 × 24 | - JAPAN |
| - G1 - G2 - G3 - G4 - G5 - G6 | - 16 - 17 - 14 - 15 - 18 - 19 | - 16 × 16 - 32 × 32 - 24 × 24 - 48 × 48 - 16 × 32 - 24 × 48 | - CHINA |
| - T1 - T2 - T3 | - 1E - 1F - 20 | - 16 x 32 - 16 x 64 - 32 x 64 | - THAILAND |
| - K1 - K2 - K3 | - 21 - 22 - 23 | - 16 × 16 - 16 × 32 - 32 × 32 | - KOREA |

2.   PLU TOTAL FILE - (26H)

- PLU NO						-	4 BYTES	(BCD)

- PLU TOTAL RECORD SIZE			-	2 BYTES	(HEX)

- EAN CODE						-	7 BYTES	(BCD)

- DUMMY						-	1 BYTE

- MAIN GROUP CODE				-	2 BYTES	(BCD)

- TOTAL DAILY					-	18 BYTES

- TOTAL ACTUAL PRICE		-	4 BYTES

- TOTAL PLANNED PRICE		-	4 BYTES

- TOTAL PCS				-	2 BYTES

- TOTAL WEIGHT			-	4 BYTES

- TOTAL PROFIT			-	4 BYTES

- TOTAL TERM					-	18 BYTES

- TOTAL ACTUAL PRICE		-	4 BYTES

- TOTAL PLANNED PRICE		-	4 BYTES

- TOTAL PCS				-	2 BYTES

- TOTAL WEIGHT			-	4 BYTES

- TOTAL PROFIT			-	4 BYTES

- TOTAL PREPACK - DAILY				-	18 BYTES

- TOTAL ACTUAL PRICE		-	4 BYTES

- TOTAL PLANNED PRICE		-	4 BYTES

- TOTAL PCS				-	2 BYTES

- TOTAL WEIGHT			-	4 BYTES

- TOTAL PROFIT			-	4 BYTES

- TOTAL PREPACK - TERM				-	18 BYTES

- TOTAL ACTUAL PRICE		-	4 BYTES

- TOTAL PLANNED PRICE		-	4 BYTES

- TOTAL PCS				-	2 BYTES

- TOTAL WEIGHT			-	4 BYTES

- TOTAL PROFIT			-	4 BYTES

- TOTAL DISCOUNT					-	18 BYTES

- TOTAL ACTUAL PRICE		-	4 BYTES

- TOTAL PLANNED PRICE		-	4 BYTES

- TOTAL PCS				-	2 BYTES

- TOTAL WEIGHT			-	4 BYTES

- TOTAL PROFIT			-	4 BYTES

- PLU LAST ACT DATE		-	3 BYTES (6 digits BCD, DD.MM.YY)

- Maximum number of departments = N

- No. of bytes occupy = N × (19 + 18 × 5) = 109N BYTES

PRINT FORMAT FILE (34H)

PRINT FORMAT NUMBER (See FORMAT # Table)	-	4 BYTES (BCD)

PRINT FORMAT RECORD SIZE			-	2 BYTES (HEX)

PRINT FORMAT WIDTH				-	2 BYTES (HEX)

PRINT FORMAT HEIGHT				-	2 BYTES (HEX)

PRINT FORMAT STATUS				-	2 BYTES (BIN)

FORMAT FOR EACH FIELD				-	1106 BYTES

PLU NO					-	8 BYTES	N1

PRICE						-	8 BYTES	N1

UNIT PRICE					-	8 BYTES	N1

WEIGHT					-	8 BYTES	N1

QUANTITY					-	8 BYTES	N1

PACKED DATE				-	8 BYTES	N1

PACKED TIME				-	8 BYTES	N1

COMMODITY NAME			-	14 BYTES	C1

QUANTITY SYMBOL			-	8 BYTES	N1

SELL BY DATE				-	8 BYTES	N1

SELL BY TIME				-	8 BYTES	N1

BARCODE					-	12 BYTES	B1

SHOP NAME					-	12 BYTES	C2

DISCOUNT VALUE				-	8 BYTES	N1

USED BY DATE				-	8 BYTES	N1

LOGO						-	12 BYTES	B2

MG CODE					-	8 BYTES	N1

DEPARTMENT CODE			-	8 BYTES	N1

SHOP INTERNAL SCALE NO.		-	8 BYTES	N1

INGREDIENTS				-	26 BYTES	C3

SPECICAL MESSAGE			-	18 BYTES	C4

ITEM PRICE AFTER TAX			-	8 BYTES	N1

ITEM TAX RATE				-	8 BYTES	N1

TOTAL PRICE BEFORE TAX		-	8 BYTES	N1

FRAME 1					-	12 BYTES	B3

FRAME 2					-	12 BYTES	B3

FRAME 3					-	12 BYTES	B3

FRAME 4					-	12 BYTES	B3

FRAME 5					-	12 BYTES	B3

FRAME 6					-	12 BYTES	B3

FRAME 7					-	12 BYTES	B3

FRAME 8					-	12 BYTES	B3

FRAME 9					-	12 BYTES	B3

FRAME 10					-	12 BYTES	B3

TARE						-	8 BYTES	N1

CLERK CODE 1				-	8 BYTES	N1

IMAGE 1					-	12 BYTES	B4

IMAGE 2					-	12 BYTES	B4

IMAGE 3					-	12 BYTES	B4

IMAGE 4					-	12 BYTES	B4

IMAGE 5					-	12 BYTES	B4

IMAGE 6					-	12 BYTES	B4

IMAGE 7					-	12 BYTES	B4

IMAGE 8					-	12 BYTES	B4

IMAGE 9					-	12 BYTES	B4

IMAGE 10					-	12 BYTES	B4

*RESERVE 1					-	8 BYTES	N1

*RESERVE 2					-	8 BYTES	N1

*RESERVE 3					-	8 BYTES	N1

*RESERVE 4					-	8 BYTES	N1

*RESERVE 5					-	8 BYTES	N1

*RESERVE 6					-	8 BYTES	N1

TOTAL LABEL PRINT FORMAT

TOTAL PLU NO.				-	8 BYTES	N1

TOTAL PACKED DATE			-	8 BYTES	N1

TOTAL WEIGHT				-	8 BYTES	N1

TOTAL PACKED QUANTITY		-	8 BYTES	N1

TOTAL QUANTITY SYMBOL		-	8 BYTES	N1

TOTAL PRICE				-	8 BYTES	N1

TOTAL PACKED TIME			-	8 BYTES	N1

ALL TOTAL					-	8 BYTES	N1

TOTAL BARCODE				-	12 BYTES	B1

CLERK CODE 2				-	8 BYTES	N1

TOTAL FRAME 1				-	12 BYTES	B3

TOTAL FRAME 2				-	12 BYTES	B3

TOTAL FRAME 3				-	12 BYTES	B3

TOTAL FRAME 4				-	12 BYTES	B3

TOTAL FRAME 5				-	12 BYTES	B3

TOTAL FRAME 6				-	12 BYTES	B3

TOTAL FRAME 7				-	12 BYTES	B3

TOTAL FRAME 8				-	12 BYTES	B3

TOTAL FRAME 9				-	12 BYTES	B3

TOTAL FRAME 10				-	12 BYTES	B3

TEXT 1					-	12 BYTES	C2

TEXT 2					-	12 BYTES	C2

TEXT 3					-	12 BYTES	C2

TEXT 4					-	12 BYTES	C2

TEXT 5					-	12 BYTES	C2

TEXT 6					-	12 BYTES	C2

TEXT 7					-	12 BYTES	C2

TEXT 8					-	12 BYTES	C2

TEXT 9					-	12 BYTES	C2

TEXT 10					-	12 BYTES	C2

TEXT 11					-	12 BYTES	C2

TEXT 12					-	12 BYTES	C2

TEXT 13					-	12 BYTES	C2

TEXT 14					-	12 BYTES	C2

TEXT 15					-	12 BYTES	C2

TEXT 16					-	12 BYTES	C2

TEXT 17					-	12 BYTES	C2

TEXT 18					-	12 BYTES	C2

TEXT 19					-	12 BYTES	C2

TEXT 20					-	12 BYTES	C2

TOTAL ADD ON TAX AMT		-	8 BYTES	N1

TOTAL VAT TAX AMT			-	8 BYTES	N1

PLACE					-	12 BYTES	C2

TOTAL IMAGE 1				-	12 BYTES	B4

TOTAL IMAGE 2				-	12 BYTES	B4

TOTAL IMAGE 3				-	12 BYTES	B4

TOTAL IMAGE 4				-	12 BYTES	B4

TOTAL IMAGE 5				-	12 BYTES	B4

AVERAGE PRICE				-	8 BYTES	N1

AVERAGE WEIGHT				-	8 BYTES	N1

BONUS					-	8 BYTES	N1

TOTAL USED BY DATE			-	8 BYTES	N1

(VERION 2.51 & ABOVE)

APPLICABLE ONLY TO PRINT FORMAT STATUS = 1

2ND QUANTITY (DP90)			-	8 BYTES	N1

DATE CODE (DP90)				-	8 BYTES	N1

CLERK NAME (DP90)			-	8 BYTES	N1

DEPT NAME (DP90)				-	8 BYTES	N1

RESERVED					-	250 BYTES

VER 2.72 & ONWARDS (Non applicable to country code AA)

Maximum number of format type = 99

BIT 1 of PRINT FORMAT STATUS = 0

Number of bytes occupied per record = 1118 BYTES

BIT 1 of PRINT FORMAT STATUS = 1

Number of bytes occupied per record = 1400 BYTES

Format of each printing item

1.	B1 Type				(12 Bytes)

X				(2 Bytes) (HEX)

X axis as the base for printing (The origin is located at the bottom left)

Y				(2 Bytes) (HEX)

Y axis as the base for printing (The origin is located at the bottom left)

STATUS			(2 Bytes) (See STATUS Table)

Itemized status for printing item

H				(2 Bytes) (HEX)

Size of printing area towards vertical direction

W				(2 Bytes) (HEX)

Size of printing area towards horizontal direction

STATUS2			(2 Bytes) (DUMMY)

Bit 0:	0: Printing readable characters

1: No readable characters

Bit 1:	0: Printing test bar code

1: No test bar code

2.	B2 Type				(12 Bytes)

X				(2 Bytes) (HEX)

X axis as the base for printing (The origin is located at the bottom left)

Y				(2 Bytes) (HEX)

Y axis as the base for printing (The origin is located at the bottom left)

STATUS			(2 Bytes) (See STATUS Table)

Itemized status for printing item

H				(2 Bytes) (HEX)

Size of printing area towards vertical direction

W				(2 Bytes) (HEX)

Size of printing area towards horizontal direction

LOGO SIZE			(2 Bytes) (HEX, FIRST BYTE)

Size of Logo	(0:None	1:Normal Size		2:Double Size)

3.	B3 Type				(12 Bytes)

X1				(2 Bytes) (HEX)

X axis as the base of the frame

Y1				(2 Bytes) (HEX)

Y axis as the base of the frame

STATUS			(2 Bytes) (See STATUS Table)

Itemized status for printing each item

X2				(2 Bytes) (HEX)

X axis on diagonal of the frame

Y2				(2 Bytes) (HEX)

Y axis on diagonal of the frame

W				(2 Bytes) (HEX)

Thickness of the frame

4.	B4 Type				(12 Bytes)

X				(2 Bytes) (HEX)

X axis as the base for printing (The origin is located at the bottom left)

Y				(2 Bytes) (HEX)

Y axis as the base for printing (The origin is located at the bottom left)

STATUS			(2 Bytes) (See STATUS Table)

Itemized status for printing item

H				(2 Bytes) (HEX)

Size of printing area towards vertical direction

W				(2 Bytes) (HEX)

Size of printing area towards horizontal direction

IMAGE NUMBER		(2 Bytes) (BCD, SECOND BYTE)

Image number assigned to this free format when SPEC selected

5.	C1 Type				(14 Bytes)

X				(2 Bytes) (HEX)

X axis as the base for printing (The origin is located at the bottom left)

Y				(2 Bytes) (HEX)

Y axis as the base for printing (The origin is located at the bottom left)

STATUS			(2 Bytes) (See STATUS Table)

Itemized status for printing each item

H				(2 Bytes) (HEX)

Size of printing area towards vertical direction

W				(2 Bytes) (HEX)

Size of printing area towards horizontal direction

CHARACTER SIZE 1	(1 Byte) (HEX)

Size of characters in the 1st line

CHARACTER SIZE 2	(1 Byte) (DUMMY)

Size of characters in the 2nd line

CHARACTER SIZE 3	(1 Byte) (DUMMY)

Size of characters in the 3rd line

CHARACTER SIZE 4	(1 Byte) (DUMMY)

Size of characters in the 4th line.

6.	C2 Type				(12 Bytes)

X				(2 Bytes) (HEX)

X axis as the base for printing (The origin is located at the bottom left)

Y				(2 Bytes) (HEX)

Y axis as the base for printing (The origin is located at the bottom left)

STATUS			(2 Bytes) (See STATUS Table)

Itemized status for printing each item

H				(2 Bytes) (HEX)

Size of printing area towards vertical direction

W				(2 Bytes) (HEX)

Size of printing area towards horizontal direction

CHARACTER SIZE 1	(1 Byte) (HEX)

Size of characters in the 1st line

CHARACTER SIZE 2	(1 Byte) (DUMMY)

Size of characters in the 2nd line

7.	C3 Type				(26 Bytes)

X				(2 Bytes) (HEX)

X axis as the base for printing (The origin is located at the bottom left)

Y				(2 Bytes) (HEX)

Y axis as the base for printing (The origin is located at the bottom left)

STATUS			(2 Bytes) (See STATUS Table)

Itemized status for printing item

H				(2 Bytes) (HEX)

Size of printing area towards vertical direction

W				(2 Bytes) (HEX)

Size of printing area towards horizontal direction

CHARACTER SIZE 1	(1 Byte) (HEX)

Size of characters in the 1st line

CHARACTER SIZE 2	(1 Byte) (DUMMY)

Size of characters in the 2nd line



CHARACTER SIZE 15	(1 Byte) (DUMMY)

Size of characters in the 15th line

DUMMY			(1 Byte)

8.	C4 Type				(18 Bytes)

X				(2 Bytes) (HEX)

X axis as the base for printing (The origin is located at the bottom left)

Y				(2 Bytes) (HEX)

Y axis as the base for printing (The origin is located at the bottom left)

STATUS			(2 Bytes) (See STATUS Table)

Itemized status for printing item

H				(2 Bytes) (HEX)

Size of printing area towards vertical direction

W				(2 Bytes) (HEX)

Size of printing area towards horizontal direction

CHARACTER SIZE 1	(1 Byte) (HEX)

Size of characters in the 1st line

CHARACTER SIZE 2	(1 Byte) (DUMMY)

Size of characters in the 2nd line



CHARACTER SIZE 8	(1 Byte) (DUMMY)

Size of characters in the 8th line

9.	N1 Type				(8 Bytes)

X				(2 Bytes) (HEX)

X axis as the base for printing (The origin is located at the bottom left)

Y				(2 Bytes) (HEX)

Y axis as the base for printing (The origin is located at the bottom left)

STATUS			(2 Bytes) (See STATUS Table)

Itemized status for printing item

CHARACTER SIZE		(1 Byte) (HEX)

Size of printing characters

Number of Digits		(1 Byte) (HEX)

Number of characters

Format #

| PRINT FORMAT NUMBER | DATA CODE (4 BYTES) |
| --- | --- |
| F1 - F8 | 00000001H – 00000008H |
| S | 00000080H |
| A | 00000081H |
| B | 00000082H |
| C | 00000083H |
| T1 - T16 | 00000084H - 00000093H |
| T21 - T28 | 00000098H - 0000009FH |

Status

| BIT 0 – 3 | NOT USED |  |  |
| --- | --- | --- | --- |
| BIT 4 | When first price base Printed with first price term |  |  |
| BIT 5 | When second price base Printed with second price term |  |  |
| BIT 6 | When Non-Weigh Item Printed with non-weighing item |  |  |
| BIT 7 – 13 | NOT USED |  |  |
| BIT 14,15 | ANGLE – Direction of characters’ expansion |  |  |
|  | Bit 15 | Bit 14 | Angle |
|  | 0 0 1 1 | 0 1 0 1 | 0º 90º 180º 270º |

Information for BIT 4 to 6 of STATUS word:

Example based on 0 degree:

0 (B'0000000000000000): no print

1 (B'0000000000010000): 1st price print only

2 (B'0000000000100000): 2nd price print only

3 (B'0000000000110000): 1st & 2nd price print only

4 (B'0000000001000000): non weigh item print

5 (B'0000000001010000): non weigh & 1st price

6 (B'0000000001100000): non weigh & 2nd price

7 (B'0000000001110000): all print

Applicable to countries that can select from 0 to 3 only for STATUS under free format programming.

0 corresponds to 0: no print

1 corresponds to 1: 1st price print only

2 corresponds to 4: non weigh item print

3 corresponds to 7: all print

Note:

- RESERVE1 is used for price before discount (PC BEF. DIS)

- RESERVE2 is used for unit price before discount (UP BEF. DIS)

- RESERVE3 is used for EURO UNIT PRICE

- RESERVE4 is used for EURO TOTAL PRICE

- RESERVE5 is used for EURO CALC. LINE

- RESERVE6 is used for TOTAL LABEL EURO TOTAL PRICE                           (SM90 Version 2.23 & Above)

- X and Y axis bytes are in DOTS instead of millimeters. Eg: 1000 dots = 135 mm

- Fields reserved but not yet catered for under free format programming are FRAME 3 to FRAME 10, TOTAL FRAME 1 to TOTAL FRAME 10, & TOTAL IMAGE 1 to TOTAL IMAGE 5.

Character Size

| Character | Data Code (HEX) | Size | Country |
| --- | --- | --- | --- |
| S1 S2 S3 S4 S5 *S6 (DP90) | 00 01 02 03 04 0E | 5 × 7 6 × 10 7 × 12 7 × 16 9 × 22 22 × 40 | STANDARD |
| M1 M2 M3 M4 M5 *M6 (DP90) | 05 06 07 08 09 0F | 10 × 14 12 × 20 14 × 24 14 × 32 18 × 44 44 × 80 | STANDARD |
| B1 B2 B3 B4 B5 B6 | 0A 0B 1A 1B 1C 1D | 24 × 24 48 × 48 24 × 48 16 × 15 16 × 30 32 × 30 | HONG KONG & TAIWAN |
| J1 J2 J3 J4 | 10 11 12 13 | 16 × 16 16 × 32 32 × 32 24 × 24 | JAPAN |
| G1 G2 G3 G4 G5 G6 | 16 17 14 15 18 19 | 16 × 16 32 × 32 24 × 24 48 × 48 16 × 32 24 × 48 | CHINA |
| T1 T2 T3 | 1E 1F 20 | 16 x 32 16 x 64 32 x 64 | THAILAND |
| K1 K2 K3 | 21 22 23 | 16 × 16 16 × 32 32 × 32 | KOREA |
| *D1 (DP90) *D2 (DP90) | 2E 2F | 25 x 64 50 x 128 | DENMARK |
| *W1 (DPS) | 30 | 11 x 28 | DPS USE |

Note:  0CH and 0DH cannot be used as Character Size because these are the section

end code and line end code.

TEXT FILE (38H)

TEXT NUMBER					-	4 BYTES (BCD)

TEXT RECORD SIZE				-	2 BYTES (HEX)

TEXT LABEL					-	2 BYTES (HEX)

TEXT NAME						-	206 BYTES (ASCII)

Maximum number of text file = 20

No. of bytes occupy = 20 × 214 = 4280 BYTES

INGREDIENT FILE (3AH)

INGREDIENT NUMBER				-	4 BYTES (BCD)

INGREDIENT RECORD SIZE			-	2 BYTES (HEX)

INGREDIENT LABEL				-	2 BYTE (HEX)

INGREDIENT NAME				-	1545 BYTES (ASCII)

Maximum number of ingredient file = 16

No. of bytes occupy = 16 × 1553 = 24848 BYTES

Note:

Starting from SM90 version 2.26, if compiling flag NBI is set to 1, the field “INGREDIENT NAME” can have a maximum of (1545 + 8652 bytes).

SPECIAL MESSAGE FILE (3BH)

SPECIAL MESSAGE NUMBER			-	4 BYTES (BCD)

SPECIAL MESSAGE RECORD SIZE		-	2 BYTES (HEX)

SPECIAL MESSAGE LABEL			-	2 BYTES (HEX)

SPECIAL MESSAGE NAME			-	824 BYTES (ASCII)

Maximum number of special message file = 16

No. of bytes occupy = 16 × 832 = 13312 BYTES

SHOP NAME FILE (3DH)

SHOP NUMBER					-	4 BYTES (BCD)

SHOP RECORD SIZE				-	2 BYTES (HEX)

SHOP LABEL						-	2 BYTES (HEX)

SHOP NAME						-	309 BYTES (ASCII)

Maximum number of shop name file = 32

No. of bytes occupy = 32 × 317 = 10144 BYTES

SCROLL MESSAGE FILE (3FH)

SCROLL MESSAGE NUMBER			-	4 BYTES (BCD)

SCROLL MESSAGE RECORD SIZE		-	2 BYTES (HEX)

SCROLL MESSAGE DATA				-	256 BYTES

Maximum number of scroll message file = 9

No. of bytes occupy = 9 × 262 = 2358 BYTES

Scroll Message Data (256 Bytes)

1st byte		Number of Characters (HEX)

2nd - 255th	Data (ASCII)

256th		Code Type (HEX)

SCROLL SEQUENCE FILE (40H)

SCROLL SEQUENCE NUMBER			-	4 BYTES (BCD)

SCROLL SEQUENCE RECORD SIZE		-	2 BYTES (HEX)

SCROLL SEQUENCE 1				-	5 BYTES

SCROLL Message Group 1 (0 – 9)	-	1 BYTE (BCD)

FILE 1 (1 – 9)				-	1 BYTE (BCD)

SCROLL 1				-	1 BYTE

DISPLAY 1				-	1 BYTE

SPEED 1				-	1 BYTE

SCROLL SEQUENCE 2				-	5 BYTES

SCROLL Message Group 2 (0 – 9)	-	1 BYTE (BCD)

FILE 2 (1 – 9)				-	1 BYTE (BCD)

SCROLL 2				-	1 BYTE

DISPLAY 2				-	1 BYTE

SPEED 2				-	1 BYTE

SCROLL SEQUENCE 3				-	5 BYTES

SCROLL Message Group 3 (0 – 9)	-	1 BYTE (BCD)

FILE 3 (1 – 9)				-	1 BYTE (BCD)

SCROLL 3				-	1 BYTE

DISPLAY 3				-	1 BYTE

SPEED 3				-	1 BYTE

Maximum number of scroll sequence = 3 (9 for SERVER / CLIENT)

No. of bytes occupy = 3 × 21 = 63 BYTES

( 9 x 21 = 189 BYTES for SERVER / CLIENT)

Note:

- SCROLL MESSAGE GROUP – used in combination with FILE

Example : SCROLL MESSAGE GROUP 1 = 5 & FILE 1 = 9

SCROLL MESSAGE NUMBER = 000059 (BCD)

(b) 	FILE 1, 2 & 3 - SCROLL MESSAGE NUMBER

(c) 	SCROLL & DISPLAY - used collectively as 2 bytes

SCROLL 1, 2, 3  -	Left Scroll	= 0

Stationary	= 1

Right Scroll	= 2

Wipe In	= 3

Blinking	= 1

DISPLAY 1, 2, 3 -	Left Scroll	= 1

Stationary	= 1

Right Scroll	= 1

Wipe In	= 1

Blinking	= 0

(d)	SPEED 1, 2, 3	 -	Slow		= 0

Normal	= 1

Fast		= 2

PRESET KEY ASSIGNMENT FILE (41H)

PRESET NUMBER					-	4 BYTES (BCD)

PRESET RECORD SIZE				-	2 BYTES (HEX)

PRESET KEY SWITCH #				-	4 BYTES

PRESET STATUS					-	1 BYTE

*PRESET CSIZE					-	1 BYTE

*PRESET NAME					-	8 + 24 BYTES

* PRESET CSIZE is added & PRESET NAME is increased from 8 BYTES

(DUMMY) to 32 BYTES with effect from Version 2.46 & above

Maximum number of Preset Key = 56

No. of bytes occupy = 56 × (19+25) = 1064 BYTES

Preset Status (1 Byte)

| Description | Bit 1 | Bit 0 |
| --- | --- | --- |
| PLU KEY | 0 | 0 |
| FUNCTION KEY | 0 | 1 |
| CLERK KEY | 1 | 0 |

Preset Name (8 Bytes)

Name to be programmed for Preset Keys

Preset Key Switch # (4 Bytes)

PLU# (6 digit of BCD code except 0) or FUNCTION# (See Function Number Table)

Function Number

| NO. | FUNCTION LIST | DETAIL |
| --- | --- | --- |
| 0 | CLEAR | Clear |
| 1 | ADVERT | Advertisement to be printed out |
| 2 | - PRICE | Fixed price discount |
| 3 | -P.PERC | Price percentage discount |
| 4 | -UPRICE | Fixed unit price |
| 5 | -U.PERC | Unit price percentage |
| 6 | PACK DT | Pack date = today date + no. of days entered |
| 7 | - PACK | Negative pack date = today date - no. of days entered |
| 8 | SELL DT | Sell by date = today date + no. of days entered |
| 9 | SYMBOL | Unit symbol |
| 10 | QTY SET | Quantity for non-weigh item |
| 11 | PRC CHG | Price change |
| 12 | REFUND | Refund item |
| 13 | SHOP NM | Shop name set |
| 14 | LOGO | Logo |
| 15 | PRN SEL | Print select |
| 16 | SUB TTL | Subtotal |
| 17 | GRD TTL | Grand total |
| 18 | PAC QTY | Packed quantity |
| 19 | PR MARK | Price mark down |
| 20 | PPER MK | Price per mark down |
| 21 | UP MARK | Up mark down |
| 22 | UPER MK | Up per mark down |
| 23 | STOR UP | Store up |
| 24 | PLACE | Place |
| 25 | FORMAT1 | Label format 1 |
| 26 | FORMAT2 | Label format 2 |
| 27 | P.NOADD | Prepack non add |
| 28 | MAIN GP | Main Group number |
| 29 | APC 3 | Auto PLU Call |
| 30 | SC SQ | Program scroll sequence |
| 31 | MANU WT | Set manual weight |
| 32 | ITEM CD | Item code |
| 33 | NET CON | Net content |
| 34 | EURO SW | Euro currency switch |
| 35 | SW KEY | Switch key |
| 36 | PR KEY | Price key |
| 37 | *REF PLU | Reference PLU |
| 38 | *PRT KEY | Print Key |
| 39 | PLU REP | PLU Call |
| 40 | FLAG CD | Flag Code |
| 41 | OP CASH | No Sale |
| 42 | TAX INF | Tax Info |
| 43 | PASSWD | Unit price override password |

Note:   (1) *REF PLU function added from VER 2.62 & above.

(2) *PRT KEY function added from VER 2.63 & above.

STORE TOTAL FILE (46H)

STORE TOTAL NUMBER				-	4 BYTES

STORE TOTAL RECORD SIZE			-	2 BYTES

STORE DAILY					-	80 BYTES

NO OF CUSTOMER			-	4 BYTES

NO OF ITEM				-	4 BYTES

ACTUAL SALE			-	4 BYTES

PLANNED SALE			-	4 BYTES

NOMINAL SALE			-	4 BYTES

DISCOUNT PRICE			-	4 BYTES

WEIGHT				-	4 BYTES

REFUND ITEM COUNT		-	2 BYTES

REFUND PRICE			-	4 BYTES

ALL PICKOUT REFUND CNT	-	2 BYTES

ALL PICKOUT REFUND PRICE	-	4 BYTES

ALL RCT CORRECT ITEM CNT	-	2 BYTES

ALL RCT CORRECT RCT CNT	-	2 BYTES

ALL RCT CORRECT PRICE	-	4 BYTES

VAT NO. OF TAXABLE ITEM	-	4 BYTES

VAT AMOUNT OF TAX		-	4 BYTES

VAT NO. OF EXEMPTED ITEM	-	4 BYTES

VAT AMT OF EXEMPTED TAX	-	4 BYTES

ATX NO. OF TAXABLE ITEM	-	4 BYTES

ATX AMOUNT OF TAX		-	4 BYTES

ATX NO. OF EXEMPTED ITEM	-	4 BYTES

ATX AMT OF EXEMPTED TAX	-	4 BYTES

STORE DAILY - HOURLY - H0 to H23		-	16 BYTES × 24

NO OF CUSTOMER			-	4 BYTES × 24

NO OF ITEM				-	4 BYTES × 24

ACTUAL PRICE			-	4 BYTES × 24

PLANNED PRICE			-	4 BYTES × 24

STORE DAILY REFUND				-	16 BYTES

NO OF CUSTOMER			-	4 BYTES

NO OF ITEM				-	4 BYTES

ACTUAL PRICE			-	4 BYTES

PLANNED PRICE			-	4 BYTES

STORE MONTHLY					-	80 BYTES

NO OF CUSTOMER			-	4 BYTES

NO OF ITEM				-	4 BYTES

ACTUAL SALE			-	4 BYTES

PLANNED SALE			-	4 BYTES

NOMINAL SALE			-	4 BYTES

DISCOUNT PRICE			-	4 BYTES

WEIGHT				-	4 BYTES

REFUND ITEM COUNT		-	2 BYTES

REFUND PRICE			-	4 BYTES

ALL PICKOUT REFUND CNT	-	2 BYTES

ALL PICKOUT REFUND PRICE	-	4 BYTES

ALL RCT CORRECT ITEM CNT	-	2 BYTES

ALL RCT CORRECT RCT CNT	-	2 BYTES

ALL RCT CORRECT PRICE	-	4 BYTES

VAT NO. OF TAXABLE ITEM	-	4 BYTES

VAT AMOUNT OF TAX		-	4 BYTES

VAT NO. OF EXEMPTED ITEM	-	4 BYTES

VAT AMT OF EXEMPTED TAX	-	4 BYTES

ATX NO. OF TAXABLE ITEM	-	4 BYTES

ATX AMOUNT OF TAX		-	4 BYTES

ATX NO. OF EXEMPTED ITEM	-	4 BYTES

ATX AMT OF EXEMPTED TAX	-	4 BYTES

STORE MONTHLY - HOURLY - H0 to H23	-	16 BYTES × 24

NO OF CUSTOMER			-	4 BYTES × 24

NO OF ITEM				-	4 BYTES × 24

ACTUAL PRICE			-	4 BYTES × 24

PLANNED PRICE			-	4 BYTES × 24

STORE MONTHLY REFUND			-	16 BYTES

NO OF CUSTOMER			-	4 BYTES

NO OF ITEM				-	4 BYTES

ACTUAL PRICE			-	4 BYTES

PLANNED PRICE			-	4 BYTES

STORE TERM					-	80 BYTES

NO OF CUSTOMER			-	4 BYTES

NO OF ITEM				-	4 BYTES

ACTUAL SALE			-	4 BYTES

PLANNED SALE			-	4 BYTES

NOMINAL SALE			-	4 BYTES

DISCOUNT PRICE			-	4 BYTES

WEIGHT				-	4 BYTES

REFUND ITEM COUNT		-	2 BYTES

REFUND PRICE			-	4 BYTES

ALL PICKOUT REFUND CNT	-	2 BYTES

ALL PICKOUT REFUND PRICE	-	4 BYTES

ALL RCT CORRECT ITEM CNT	-	2 BYTES

ALL RCT CORRECT RCT CNT	-	2 BYTES

ALL RCT CORRECT PRICE	-	4 BYTES

VAT NO. OF TAXABLE ITEM	-	4 BYTES

VAT AMOUNT OF TAX		-	4 BYTES

VAT NO. OF EXEMPTED ITEM	-	4 BYTES

VAT AMT OF EXEMPTED TAX	-	4 BYTES

ATX NO. OF TAXABLE ITEM	-	4 BYTES

ATX AMOUNT OF TAX		-	4 BYTES

ATX NO. OF EXEMPTED ITEM	-	4 BYTES

ATX AMT OF EXEMPTED TAX	-	4 BYTES

STORE TERM - HOURLY - H0 to H23		-	16 BYTES × 24

NO OF CUSTOMER			-	4 BYTES × 24

NO OF ITEM				-	4 BYTES × 24

ACTUAL PRICE			-	4 BYTES × 24

PLANNED PRICE			-	4 BYTES × 24

STORE TERM REFUND				-	16 BYTES

NO OF CUSTOMER			-	4 BYTES

NO OF ITEM				-	4 BYTES

ACTUAL PRICE			-	4 BYTES

PLANNED PRICE			-	4 BYTES

STORE LAST MONTHLY				-	80 BYTES

NO OF CUSTOMER			-	4 BYTES

NO OF ITEM				-	4 BYTES

ACTUAL SALE			-	4 BYTES

PLANNED SALE			-	4 BYTES

NOMINAL SALE			-	4 BYTES

DISCOUNT PRICE			-	4 BYTES

WEIGHT				-	4 BYTES

REFUND ITEM COUNT		-	2 BYTES

REFUND PRICE			-	4 BYTES

ALL PICKOUT REFUND CNT	-	2 BYTES

ALL PICKOUT REFUND PRICE	-	4 BYTES

ALL RCT CORRECT ITEM CNT	-	2 BYTES

ALL RCT CORRECT RCT CNT	-	2 BYTES

ALL RCT CORRECT PRICE	-	4 BYTES

VAT NO. OF TAXABLE ITEM	-	4 BYTES

VAT AMOUNT OF TAX		-	4 BYTES

VAT NO. OF EXEMPTED ITEM	-	4 BYTES

VAT AMT OF EXEMPTED TAX	-	4 BYTES

ATX NO. OF TAXABLE ITEM	-	4 BYTES

ATX AMOUNT OF TAX		-	4 BYTES

ATX NO. OF EXEMPTED ITEM	-	4 BYTES

ATX AMT OF EXEMPTED TAX	-	4 BYTES

STORE LAST MONTHLY - HOURLY (H0-H23)	-	16 BYTES × 24

NO OF CUSTOMER			-	4 BYTES × 24

NO OF ITEM				-	4 BYTES × 24

ACTUAL PRICE			-	4 BYTES × 24

PLANNED PRICE			-	4 BYTES × 24

STORE LAST MONTHLY REFUND		-	16 BYTES

NO OF CUSTOMER			-	4 BYTES

NO OF ITEM				-	4 BYTES

ACTUAL PRICE			-	4 BYTES

PLANNED PRICE			-	4 BYTES

STORE WORKING DAYS				-	2 BYTES

(No. of operating days of the month)

STORE LAST MONTH WORKING DAYS		-	2 BYTES

(No. of operating days of last month)

PLU TOTAL TRANSACTION NO			-	4 BYTES (BCD)

Maximum number of store = 1

No. of bytes occupy = 1 × 1930 = 1930 BYTES